GLIDER WINCHES

E11-E14 Wem Industrial Estate,

Wem, Shropshire, SY4 5SD ENGLAND

Tel:  +44(0) 1939 235845

Fax: +44(0) 1939 234059

Website: www.skylaunchuk.com

Email: skylaunchwinches@yahoo.co.uk

Skylaunch Control Panel with presets for Glider type and headwind component:

A

B

C

1

2

Blue Colour-Coded (Launch) Levers-

A) Transmission Select Lever (N>D)

B) Engine Throttle Lever

C) Drums Brake Lever (both Drums)

Black Throttle Pre-selects-

1) Headwind Speed

2) Glider Type

(See attached Throttle Setting Chart)

For safety, all controls are designed to operate as push to go, and pull to stop.

1

2

Before Launch (Black Levers):

Preselect Headwind Speed (1)

Preselect Glider type to be launched (2)

Select left or right Drum

To Tighten Cable (Blue Lever):

Select Transmission Lever to D Position (A)

To Launch (Blue Levers):

Apply Throttle to preselected guide stop (B)

Throttle damper controls correct acceleration rate