SKYLAUNCH RE-ENGINEERING QUOTATION

Winch:_Tost Winch for Planeurs d’ille-et-Vilaine	Date:___24 / 10 / 2011

| Subject | Description | Fitted/ Condition | Recommendations |
| --- | --- | --- | --- |
| A) Final Drive | 1) Design (upside down or right way up?) | Upside down | Turn over, modify and re-engineer mounts, linkages etc. |
|  | 2) Brakes for cable drums | None | Skylaunch disk brake conversion |
|  | 3) Drive shafts | Usable | Service/ check over |
|  | 4) General condition | Usable | Service/ check over – overhaul if necessary |
|  | 5) Cable pay-on (Level wind) | Already removed | Not needed- Narrow drums |
| B) Cable drums | 1) Original drums condition/ design/ diameter | None | Modify Chassis + fit new drums |
|  | 2) Suitable for Skyrope/ Dyneema/ Steel Cable | Yes | Upgrade with additional webs + catch rails |
|  | 3) Safety drum covers | None | Manufacture new |
| c) Engine | 1) Type and capacity | 7.4 litre GM V8 | Relocate engine position with (Marine) rubber mounts |
|  | 2) Condition | Good | Service/ check over/ update Option for new 8.2 Litre GM V8 |
|  | 3)Fuel type/ system | Petrol | Original carburettor petrol or conversion to fuel injection or LPG system |
|  | 4) Exhaust system | Usable | Reuse/overhaul existing tubular system or fit Skylaunch system |
|  | 5) Cooling | Usable | Service/ check over, fit engine fans as required |
|  | 6)Fuel tank | None | Fit new/ used petrol or LPG tank |
|  | 7) Wiring/ plumbing | Poor | Rewire and renew pipework |
| D) Trans- mission | 1) Type | GM T400 Automatic | Reuse but recondition |
|  | 2) Condition | Usable but locked in 3rd | Rebuild to use all 3 gears |
|  | 3) Cooling | Poor | Fit new fan assisted cooler with thermostatic control |
|  | 4) Prop shaft | Usable | Service/ overhaul + lengthen |
|  | 5) Prop shaft brake | None | Fit new system |
| E) Cable guillotines | 1) Design/ performance | Original Tost | Re-engineer with increased power for Skyrope or renew with Skylaunch Chisel Guillotines for use with dyneema |
|  | 2) Condition | Good | Re-engineer with increased power |
|  | 3) Actuation/ linkage | Poor | Renew with heavy duty system |
| F) Cable Roller box | 1) Design | Pulley/ roller type | Already overhauled by Skylaunch – suitable for use with Skyrope or renew with Skylaunch swivelling Pulley/Rollerbox system for Dyneema use. |
|  | 2) Condition | Very Good | Add counter balance weight |
| G) Controls | 1) Design | Poor | Re-engineer |
|  | 2) Launch control system | None | Fit Skylaunch control system |
|  | 3) Wiring, instruments, switchgear | Old | Renew or reuse some instruments and renew all wiring etc |
|  | 4) Controls, levers, etc. | Poor | Renew/ re-engineer |
| H) Cabin | 1) Design | Poor, no safety protection | Manufacture new, wider (1.1m) basic cabin with weld mesh screens |
|  | 2) Condition | Unsafe | See above |
|  | 3) Open/ closed | Open | Option for closed cabin with laminated safety glass and rear door |
|  | 4)Seat | None | Fit padded seat |
| I) Chassis | 1)Design | Usable | Re-use/ Modify |
|  | 2) Subframe, twin axles, wheels, hitch etc for highway towing | None | Fit all new |
| J) Bodywork | 1) Panels, safety covers | Most usable | New drum covers for new drums and other new guards, panels etc. |
|  | 2) Paint/ powder | Usable but poor | Respray in Skylaunch High Visibility Blue and Yellow |

| K) Prices |  |  |
| --- | --- | --- |
| Serial | Engine Option | Price |
| 1) | Powered by Original 7.4 Litre Engine on Petrol (Carburettor) | ### £33,926.00 UK Pounds |
| 2) | Powered by Original 7.4 Litre Engine on Petrol (Fuel Injection) | ### £36,754.00 UK Pounds |
| 3) | Powered by Original 7.4 Litre Engine on LPG | ### £35,615.00 UK Pounds |
| 4) | Powered by all new GM 8.2 Litre Engine on LPG (Propan) 340 HP | ### £41,686.00 UK Pounds |
| 5) | Powered by all new GM 8.2 Litre Engine on Petrol (Fuel Injection) 400 HP | ### £43,664.00 UK Pounds |
| Price also includes delivery and full training in operation and maintenance at Gliding Club, assumes the Club provides food and accommodation for 1-2 persons for estimated 2 days. |  |  |

| Options |  |  |
| --- | --- | --- |
| Serial | Option | Price |
| L) | Enclosed Basic Cabin with 1 rear door and laminated safety Glass. | ADD £2,360.00 |
| M) | EITHER: 9.6mm Skyrope 900m fitted to both Drums (ie. 1800m total fitted to Winch) | ADD £432.00 |
| N) | OR: 5mm Dyneema Launch Cable 900m fitted to both Drums (ie. 1800m total fitted to Winch) Longer lengths of Dyneema available up to 3000m per Drum to suit length of Airfield. | ADD £1,926.00 |
|  | Note: If using Dyneema, Tost guillotines and cable roller heads will need to be replaced with 2 x all new Skylaunch Pulley / roller units and high powered chisel Guillotine systems | ADD £4,406.00 |
| O) | Launching Equipment Complete “Starter” Package for Twin Drum Launching– 2 x 17 metre rope traces including Stainless Steel ring and quick release hook. 4 x Heavy duty intermediate parachutes 5 x 3 Metre rope strops including Glider Rings, Stainless Steel Ring and braided Hose. 5 x Sets of Plastic H Section weak link housings, shackles weak links, parachute connections, quick release rings etc, etc. | ADD £700.00 |

ALL PRICES EXCLUDE VAT, taxes and duties, etc. If no VAT number is supplied, UK VAT rate is 20%.

Prices quoted are valid until 31st March 2012

Skylaunch have a €Euro account but have quoted in £Pounds due to exchange rate fluctuations. Exact Euro rates for Invoicing can be advised at time of payment if required.

Option (O) for use with Dyneema – Skylaunch Pulley / roller units and high powered chisel Guillotine systems (Below)

Examples of similar style Cabins. Note: Cabin quoted would be wider than those shown below.

##### RE-ENGINEERED BY SKYLAUNCH









