GLIDER WINCHES

E11-E14 Wem Industrial Estate,

Wem, Shropshire, SY4 5SD ENGLAND

Tel:  +44(0) 1939 235845

Fax: +44(0) 1939 234059

Website: www.skylaunchuk.com

Email: skylaunchwinches@yahoo.co.uk

SKYLAUNCH RE-ENGINEERING QUOTATION

Winch: Tost Winch for Planeurs d’Orbs	Date:___28 / 11 / 2011

| Subject | Description | Fitted/ Condition | Recommendations |
| --- | --- | --- | --- |
| A) Final Drive | 1) Design (upside down or right way up?) | Upside down | Turn over, modify and re-engineer mounts, linkages etc. |
|  | 2) Brakes for cable drums | None | Skylaunch disk brake conversion |
|  | 3) Drive shafts | Usable | Service/ check over |
|  | 4) General condition | Usable | Service/ check over – overhaul if necessary |
|  | 5) Cable pay-on (Level wind) | Already removed | Not needed- Narrow drums |
| B) Cable drums | 1) Original drums condition/ design/ diameter | None | Modify Chassis + fit new drums |
|  | 2) Suitable for Skyrope/ Dyneema/ Steel Cable | Yes | Upgrade with additional webs + catch rails |
|  | 3) Safety drum covers | None | Manufacture new |
| c) Engine | 1) Type and capacity | 7.4 litre GM V8 | Relocate engine position with (Marine) rubber mounts |
|  | 2) Condition | Good | Service/ check over/ update. Option for new 8.2 Litre GM V8 upgrade in the future |
|  | 3)Fuel type/ system | Petrol | Original carburettor petrol or conversion to fuel injection or LPG system |
|  | 4) Exhaust system | Usable | Reuse/overhaul existing tubular system or fit Skylaunch system |
|  | 5) Cooling | Usable | Service/ check over, fit engine fans as required |
|  | 6)Fuel tank | None | Fit used petrol tank |
|  | 7) Wiring/ plumbing | Poor | Rewire and renew pipework |
| D) Trans- mission | 1) Type | GM T400 Automatic | Reuse but recondition |
|  | 2) Condition | Usable but locked in 3rd | Rebuild to use all 3 gears |
|  | 3) Cooling | Poor | Fit new fan assisted cooler with thermostatic control |
|  | 4) Prop shaft | Usable | Service/ overhaul + lengthen |
|  | 5) Prop shaft brake | None | Fit new system |
| E) Cable guillotines | 1) Design/ performance | Original Tost | Re-engineer with increased power for Skyrope |
|  | 2) Condition | Good | Re-engineer with increased power |
|  | 3) Actuation/ linkage | Poor | Renew with heavy duty system |
| F) Cable Roller box | 1) Design | Pulley/ roller type | Already overhauled by Skylaunch – suitable for use with Skyrope |
|  | 2) Condition | Very Good | Add counter balance weight |
| G) Controls | 1) Design | Poor | Re-engineer |
|  | 2) Launch control system | None | Fit Skylaunch control system |
|  | 3) Wiring, instruments, switchgear | Old | Renew or reuse some instruments and renew all wiring etc |
|  | 4) Controls, levers, etc. | Poor | Renew/ re-engineer |
| H) Cabin | 1) Design | Poor, no safety protection | Manufacture new, wider (1.1m) Enclosed basic cabin with 1 wide rear door and Laminated safety glass |
|  | 2) Condition | Unsafe | See above |
|  | 3) Open/ closed | Open | Option for closed cabin with laminated safety glass and rear door |
|  | 4)Seat | None | Fit removable (for wheelchair) padded seat |
| I) Chassis | 1)Design | Usable | Re-use/ Modify |
|  | 2) Subframe, twin axles, wheels, hitch etc for highway towing | None | Fit all new |
| J) Bodywork | 1) Panels, safety covers | Most usable | New drum covers for new drums and other new guards, panels etc. |
|  | 2) Paint/ powder | Usable but poor | Respray in Skylaunch High Visibility Blue and Yellow |
| K) Cables etc. | 1) Launch Cables | None | 9.6mm Skyrope 900m fitted to both drums (ie. 1800m total) |
|  | 2) Launch equipment | None | Complete “Starter” Package for Twin Drum Launching– 2 x 17 metre rope traces including Stainless Steel ring and quick release hook. 2 x Heavy duty intermediate parachutes 5 x 3 Metre rope strops including Glider Rings, Stainless Steel Ring and braided Hose. 5 x Sets of Plastic H Section weak link housings, shackles weak links, parachute connections, quick release rings etc, etc. |
| L) Access | 1) Wheelchair Lift | None | Fit a wheelchair lift to the back of the Winch to raise a platform from the ground to a height of 1.2m, wider access and rear door. Removable seat for when wheelchair is in use. |
| M) Comms Radio | 1) Radio | None | VHF AM Airband radio Icom IC-A110 (Fitted) complete installed with: brackets/mounts, Headset, Antenna, etc |

| N) | Prices |  |  |
| --- | --- | --- | --- |
| Price – UK VAT rate is currently 20% |  | ### Price Excluding VAT | ### Price including VAT |
| Powered by Original 7.4 Litre Engine on Petrol (Carburettor) |  | ### £48,992.00 UK Pounds | ### £58,790.40 UK Pounds |
| Price in € Euros At current Exchange Rate. Note: Rate may differ at exact time of order and final payment. |  | ### €57,320.64 | ### €68,784.77 |
| Price also includes delivery and full training in operation and maintenance at Gliding Club, assumes the Club provides food and accommodation for 1-2 persons for estimated 2 days. |  |  |  |

ALL PRICES EXCLUDE VAT, taxes and duties, etc. If no VAT number is supplied, UK VAT rate is 20%.

Prices quoted are valid until 31st March 2012

Skylaunch have a €Euro account but have quoted in £Pounds due to exchange rate fluctuations. Exact Euro rates for Invoicing can be advised at time of payment if required.

##### RE-ENGINEERED BY SKYLAUNCH









